- Diagnosing Hardware Failures Theory

- List 5 common hardware issues in a desktop PC and write the typical symptom for each?

Answer:

| Hardware Issue | Typical Sympton |
| --- | --- |
| - RAM problem | PC may not boot or may give beep sounds. |
| - Power supply problem | PC does not turn on or shuts down randomly. |
| - CPU overheating | PC becomes slow or suddenly shuts down. |
| - Hard disk/SSD problem | Windows loads slowly or files fail to open. |
| - Graphics card problem | Blank screen or distorted display. |

- A laptop is not charging even when plugged in. Write a step-by-step troubleshooting plan using basic tools?

Answer:

- Check the wall socket – Make sure the socket is supplying power.

- Check the charger cable – Look for cuts, loose wires, or physical damage.

- Check the adapter – Check whether the adapter is overheating or damaged.

- Check charger output with a multimeter – Measure the DC voltage and compare it with the value written on the charger.

- Check the laptop charging port – Look for a loose or damaged charging connector.

- Check the battery – If possible, remove and reconnect the battery and check whether the laptop works directly with the charger.

- Use a multimeter to test the voltage output of a standard USB phone charger?

Answer:

I set the multimeter to DC voltage and tested the USB charger output.

Measured Voltage: 5.08 V

The normal USB voltage range is 4.75 V to 5.25 V. Therefore, 5.08 V is within the expected range and the charger is working properly.

- A PC is randomly shutting down. Use a multimeter to test the power supply's 12V rail?

Answer:

- Disconnect the ATX connector from the motherboard.

- Use a proper PSU tester or instructor-approved method to power the PSU for testing.

- Set the multimeter to DC voltage.

- Put the black probe on a black ground wire.

- Put the red probe on the yellow 12V wire.

- Record the voltage shown on the multimeter.

Measured Voltage: 11.2 V

Since 11.2 V is below 11.5 V, it may indicate a weak or faulty power supply (PSU), which can cause the PC to shut down randomly.

Safety: Do not open the PSU casing while testing.