- Component – Specific Troubleshooting Practical

- Open your desktop or laptop, locate the RAM slots, and remove and re-insert the RAM module to simulate reseating — then document each step you took and any issues you observed during the process?

Answer:

- I shut down the computer and disconnected the power supply.

- I opened the back panel/cabinet and located the RAM slot.

- I released the clips and carefully removed the RAM module.

- I checked the RAM and slot for dust or loose contact.

- I inserted the RAM again properly until it was locked in place.

- I turned on the computer and checked whether it booted normally.

Observation:
The computer started normally after reseating the RAM. No major issue was found.

- Create a troubleshooting checklist for diagnosing a faulty GPU in a gaming PC, including at least 5 steps from initial symptom identification to confirming the fix?

Answer:

| Step | Check |
| --- | --- |
| 1 | Check for blank screen, screen flickering, artifacts, or crashes |
| 2 | Check the monitor and display cable connection |
| 3 | Check whether the GPU is properly seated in the PCIe slot |
| 4 | Check the GPU power cables |
| 5 | Update or reinstall the graphics driver |
| 6 | Test the GPU in another PC or use another working GPU |
| 7 | Restart the PC and confirm that the display works normally |

- Simulate a storage issue by unplugging and replugging a SATA or NVMe drive, then describe the error messages or BIOS indicators you see before and after reconnecting?

Answer:

I disconnected the storage drive and started the computer.

- Before reconnecting:
The BIOS did not detect the drive and showed a message such as “No Boot Device Found.”

I turned off the computer and connected the drive again properly.

After reconnecting:
The drive appeared again in BIOS and the computer booted normally into Windows.

- Given a scenario where a PC randomly restarts, list which component (RAM, GPU, storage) you would check first and explain your reasoning, then outline the specific diagnostic steps?

Answer:

I would check the RAM first because loose or faulty RAM can cause random restarts, crashes, and boot problems.

- Diagnostic Steps:

- Shut down the PC and disconnect the power.

- Remove the RAM and check it for dust or damage.

- Clean the RAM contacts carefully if required.

- Reseat the RAM properly.

- If there are multiple RAM sticks, test them one at a time.

- Start the PC and check whether the random restarting problem is fixed.

- Conclusion:
If the PC works normally after reseating or replacing the RAM, the RAM was most likely causing the problem.