- Hardware Troubleshooting

- A user reports their laptop won't power on even when plugged in. List 5 troubleshooting steps you would take to diagnose the cause, and for each step, explain what you are checking for?

- Laptop Won’t Power On Even When Plugged In

If a user reports that their laptop is not powering on even when connected to the charger, I would follow these troubleshooting steps:

Step 1: Check the Power Adapter and Cable

I would check whether the charger is properly connected to both the laptop and the power socket. I would also inspect the charger and cable for any physical damage.

Checking for: A damaged charger, loose connection, or faulty power cable.

Step 2: Test the Power Outlet

I would connect another working device to the same power outlet or try charging the laptop from another socket.

Checking for: Whether the wall socket or power strip is faulty.

Step 3: Check Charging Indicator Lights

I would look for the charging LED on the laptop when the charger is connected.

Checking for: If the charging light does not appear, there may be a problem with the charger, charging port, battery, or motherboard.

Step 4: Perform a Power Reset

I would disconnect the charger and remove the battery if it is removable. Then I would press and hold the power button for about 15–30 seconds. After that, I would reconnect the charger and try turning the laptop on.

Checking for: Any temporary electrical or power-management issue.

Step 5: Test Without the Battery

If the laptop has a removable battery, I would remove the battery and try starting the laptop using only the AC adapter.

Checking for: If the laptop starts without the battery, the battery may be faulty. If it still does not start, the charger, charging port, or internal hardware may need further testing.

- You receive a helpdesk ticket: 'My monitor is showing a blank screen, but the CPU seems to be running.' Write a step-by-step troubleshooting flow you would follow to identify and resolve the issue.<br><br><em><strong>Hint:</strong> Consider checking both hardware connections and basic display settings.</em>?

- Monitor Shows a Blank Screen but CPU Is Running

I would follow this troubleshooting flow:

Step 1: Check Monitor Power

First, I would confirm that the monitor is switched on and its power indicator light is working.

Step 2: Check Display Cable

I would check whether the HDMI, DisplayPort, VGA, or other display cable is securely connected to both the monitor and computer.

Step 3: Check the Correct Input Source

Using the monitor buttons, I would make sure the correct input source such as HDMI 1, HDMI 2, DisplayPort, or VGA is selected.

Step 4: Reconnect the Display Cable

I would disconnect and reconnect the display cable from both ends to remove any loose connection.

Step 5: Test Another Cable or Port

I would try another display cable or another display port on the computer.

Step 6: Check Graphics Card Connection

If the PC has a dedicated graphics card, I would make sure the monitor cable is connected to the graphics card output instead of the motherboard display port when required.

Step 7: Test the Monitor

I would connect the monitor to another computer or laptop.

If the monitor works with another computer, the problem is likely with the original PC.

Step 8: Restart and Check Display Settings

I would restart the PC and check the Windows display settings, resolution, refresh rate, and whether the correct display is selected.

Step 9: Check Internal Hardware

If the screen is still blank, I would check the RAM and graphics card connections by safely reseating them.

Step 10: Confirm the Fix

After identifying the faulty cable, port, monitor, GPU connection, or display setting, I would correct it and restart the system to confirm that the display works normally.

- A user complains that their wireless mouse is unresponsive, but the keyboard works fine. Describe how you would isolate whether the issue is with the mouse, the USB port, or the system settings?

- Wireless Mouse Is Unresponsive but Keyboard Works

To identify whether the problem is with the mouse, USB port, or system settings, I would perform the following tests:

Step 1: Check the Mouse

I would make sure the mouse is switched ON and check or replace its batteries.

If the mouse has an indicator light, I would check whether it turns on.

Step 2: Reconnect the USB Receiver

I would remove the wireless USB receiver and reconnect it properly.

This checks whether the receiver had a loose connection.

Step 3: Test Another USB Port

I would connect the mouse receiver to another USB port.

If the mouse starts working, the original USB port may be faulty.

Step 4: Test the Mouse on Another Computer

I would connect the wireless mouse and receiver to another computer.

If the mouse still does not work, the mouse or its USB receiver is probably faulty.

If it works on another computer, the problem is most likely related to the original computer.

Step 5: Test Another Mouse

I would connect another working mouse to the original computer.

If the second mouse works properly, the original wireless mouse is faulty.

Step 6: Check System Settings and Drivers

I would open Device Manager → Mice and other pointing devices and check for driver errors or warning symbols.

I would also update or reinstall the mouse/USB drivers if necessary.

By performing these tests, I can isolate whether the issue is caused by the wireless mouse, USB receiver/port, or system drivers/settings.

- Imagine you are working on a helpdesk team for a gaming cafe. One system is overheating and shutting down during use. List 3 possible hardware causes and describe how you would check each one.<br><br><em><strong>Hint:</strong> Think about fans, dust, and thermal paste.</em>?

- Gaming PC Is Overheating and Shutting Down

Three possible hardware causes are:

1. Faulty or Non-Working Cooling Fan

The CPU, GPU, or case fan may have stopped working or may be spinning too slowly.

How I would check:

- Shut down the computer and open the case safely.

- Turn the system on and visually check whether all cooling fans are spinning.

- Check fan speeds in BIOS or hardware-monitoring software.

- Replace any fan that is not working correctly.

2. Dust Blocking Airflow

Heavy dust can collect inside the CPU heatsink, GPU, air vents, and fans, preventing proper airflow.

How I would check:

- Shut down and unplug the PC.

- Open the computer case.

- Inspect the fans, vents, heatsinks, and GPU for dust.

- Clean the dust carefully using compressed air.

After cleaning, I would monitor the temperatures again.

3. Dried or Incorrectly Applied Thermal Paste

Thermal paste transfers heat from the CPU or GPU to the heatsink. Old or dried thermal paste can cause very high temperatures.

How I would check:

- Monitor CPU temperature using BIOS or temperature-monitoring software.

- If the temperature rises very quickly even though the fan is working, I would inspect the CPU cooler.

- Remove the heatsink safely, clean the old thermal paste, apply a small amount of fresh thermal paste, and reinstall the cooler correctly.

- Test the PC under load again.

After checking these three areas, I would run the gaming PC and monitor the CPU and GPU temperatures to confirm that the overheating and shutdown problem has been resolved.